# Task List — Trajectory Clustering / Cycle Time / Idle Detection Paper

Tracks work needed to take draft_paper.md from outline to submittable draft.

## A. Data engineering (do first — everything else depends on it)

- ☐ Consolidate gps_data.csv + monthly files into one cleaned dataset; resolve column-order mismatch (gps_data_2025-7.csv has parking/alt swapped vs. other files — check before concatenating).

- ☐ Deduplicate pings, parse timestamps, sort by tracker_id, get_time.

- ☐ Compute per-ping dt (sec since last ping) and dist (haversine) per tracker; flag implausible-speed jumps.

- ☐ QA pass on zone_list.csv polygons against actual GPS density (do trucks actually cluster inside the drawn polygons, or are some zones mis-drawn/outdated?).

- ☐ Decide eps/min_samples search range for DBSCAN based on actual zone polygon sizes (compute typical zone diameter from zone_detail_all_df.csv).

## B. Site/dispatch data request (parallel track, not blocking GPS-only analysis)

- ☐ Send the data request from our last conversation to the mine site contact: payload-per-cycle, fuel logs/sensor feed, truck spec sheet (see prior message for exact field list).

- ☐ If dispatch logs arrive, write a join script keyed on tracker_id + nearest timestamp to merge into the GPS-derived cycle table.

## C. Methodology implementation

- ☐ Build stop-detection DBSCAN pipeline (Section 4.2) — start from existing zone-join logic in GPS_trackpoint_route_analysis.ipynb, replace/extend the K-means route-shape clustering with DBSCAN stop clustering.

- ☐ Label DBSCAN clusters by zone overlap; quantify the share of stops that fall outside any surveyed zone (unplanned idle).

- ☐ Implement cycle extraction (Section 4.3): load-zone stop → dump-zone stop → load-zone stop, with phase decomposition.

- ☐ Implement segment classifier (Section 4.4): transit / queuing / operating / unplanned-idle.

- ☐ Pull DEM grade along each haul segment (dem_data_get.ipynb already produces dem_merged.tif — add a sampling step along route lines).

- ☐ Run DBSCAN parameter sensitivity sweep; log how cycle count / idle % shifts.

## D. Analysis / results

- ☐ Fleet-wide time-state breakdown (transit/queue/operate/idle) with CIs.

- ☐ Cycle time distributions by truck and by load↔dump zone pair.

- ☐ Identify top bottleneck load zones (highest queue dwell).

- ☐ Map unplanned-idle hotspots.

- ☐ Trend state-time breakdown across Jul–Nov 2025 (day-of-week, shift).

- ☐ Validate a sample of detected cycles visually (use existing plot_zones_with_tracker_paths).

## E. Writing

- ☐ Fill in Results (Section 5) from D above — replace all numeric placeholders.

- ☐ Write Abstract last, once headline numbers exist.

- ☐ Write Conclusion with concrete recommendation tied to the biggest bottleneck found.

- ☐ Decide target venue (mining engineering journal vs. GIS/spatial-analytics journal vs. conference) — affects formatting, length limits, and whether queuing-theory (Section 2) or ML-comparison (vs. Fan et al. [1], Zhao et al. [7]) framing should be emphasized.

- ☐ Have an advisor/mine-engineering reader sanity-check the cycle/zone definitions in Section 4 before running full analysis — cheaper to fix definitions now than to rerun after Results are written.

## F. Optional extensions (only if B succeeds)

- ☐ Add payload-normalized productivity (tonnes/cycle-hour) once dispatch data is joined.

- ☐ Add fuel-cost-of-idle estimate once fuel data is joined — this is usually the number that gets management attention, worth prioritizing if any one extension is pursued.